James Allworth | 312073038 | Attitude Determination & Control

Thomas Forbutt | 312101058 | Communications and Data Handling

Oscar McNulty | 312106130 | Structural Design & Development

Penelope Player | 312106718 | On-Board Computer and Power System

Nikita Sardesai | 312088205 | Thermal Systems and Payload

group e: snapsat

Assembly integration and test plan

Critical Design Review
Data Package Document 3

Table of Contents

1.	Scope	2

2.	Tools	2

3.	Satellite Layout 	2

4.	Satellite Assembly	4

# Scope

The purpose of the document is to be able to assemble the CubeSat by following the procedure in this document. Therefore, it should be very precise and detailed. However, this could only be achieved after a few iterations with physical components of the CubeSat –using either mock up or actual flight hardware.

The information required in this document as part of the QB50 CDR Data Package should be used as a starting point for the assembly of the CubeSat that could (and should) be expanded upon to include more details in the upcoming revisions once the actual building of the CubeSat starts.  The changes in every revision should be indicated clearly in the Table on page 2 of this document.

The sections included in this document are kept as generic as possible as each CubeSat will be unique.  The sections are meant to be used as guidelines. Therefore, please feel free to update or modify the sections to suit the needs of your CubeSat, while conveying all the information that is required. The document should include all the tools that will be necessary during the assembly of the CubeSat, the methodology during integration and a thorough outline of the assembly procedure.

# Tools

Outlined in the table below are all the tools which will be required to build SnapSat, they are broken down into the subsystems.

| Subsystem | Tools Required |
| --- | --- |
| Thermal | Kapton tape Temperature sensors |
| Attitude Determination and Control | Copper wire (length) Winding machine |
| On-Board Computer | Components here |
| Power System | Components here |
| Communication and Data Handling | Components here |
| Structure and Integration | Components here |

# Satellite Layout

This section will give and overview on the general layout of all the internal and external components of the satellite. Also included are Solidworks parts drawings and detailed assembly drawings – along with a full 3D CAD model.

Please describe the satellite layout both internally and externally and also include CAD models (or hardware) to help with the visualization.  An example is provided below.

Layout of Main Tray and –Z Panel

The main tray layout houses the radios as well as the secondary payload as shown in Figure 1a. The secondary payload is attached to the main tray via 5 screws. The S-band enclosure is housed within the main tray itself. The UHF enclosure not only contains the UHF Rx but also serves as a shield for the S-band Tx. UHF enclosure is attached to the S-band enclosure using four screws: two screws were positioned next to the S-band SMA connectors while the other two were positioned at the back end of the UHF enclosure, also serving to secure the UHF cover. This second pair of screws passes through the UHF cover and enclosure and screws into the main tray behind the S-band.

The internal layout of the -Z panel is organized in different layers as shown in Figure 1b.  Attached closest to the panel itself are the separation switches, magnetometer, and the Z magnetorquer. The second layer is the power system, which is mounted to the panel through nine size zero screws. The HKC was to be mounted above the power system.

Figure1a:  Main tray layout 				Figure 1b:  -Z panel layout

Layout of Other Internal Components

The only other internal components are the X and Y magnetorquers which are mounted to the -X and -Y panels, respectively.

Layout of External Components

The external layout of the satellite consists of solar cell coupons, deployable UHF antennas, test port and S-band patch antennas which may be seen in Figure 2.  Two of the long faces have two solar coupons each while the remaining four faces each have only one coupon. Both the main tray and the -Z panel have two deployable UHF antennas. In addition to the UHF antenna, the -Z panel also has the test port. The S-band patch antennas are on the -X panels towards the -Y side of the satellite.

Figure 2: External layout

# Satellite Assembly

Please include a description of the assembly procedure of the CubeSat at a spacecraft level. It should also be accompanied by a CAD model indicating how the integration will take place.  An example is provided below.

- Assemble main tray:

- Fasten battery

- Fasten S-band radio

- Assembly UHF radio

- Mount UHF enclosure/S-band radio cover

- Fasten UHF radio cover.

- Fasten the -Y solar panel to main tray

- Assemble -Z panel:

- Mount separation switches, magnetometer, and Z magnetorquer;

- Fasten power system

- Mount HKC and HKC attachment plate

- Fasten -Z panel to -Y solar panel

- Integrate -X solar panel (might need temporary supports to hold the -Z panel and the

main tray in place)

- Remove -Y panel:

- Connect S-band coaxial cables to patch antenna on the -X panel.

- Integrate +X solar panel:

- Connect S-band coaxial cables to patch antenna on the +X panel

- Fasten -Y panel to -Z panel and main tray.

- Integrate secondary payload

- Integrate drag sail modules

- . Integrate +Y panel

Figure 3: Spacecraft-level assembly procedure

Some time after assembling all panel structures (but before putting in the solar panels) we need to wrap the structure in kapton tape.